Computer Architecture and Organization

Comprehensive Structured Lecture Notes • Semester III Department of Information Technology

📌 COURSE SYLLABUS & METADATA OVERVIEW Course & Instructor Details: Sem III Dept. IT ¦ Course Code: ITC304 ¦ Lecture Notes by Mrs. Anuprita Rane (Dept. of EXTC). Prerequisite: Digital Logic Design (Boolean algebra, logic gates, adders, flip-flops, multiplexers). Curriculum Coverage (Unit 1 - Overview): 1. Introduction of COA, 2. Basic Organization of Computer, 3. Block Level Description of Functional Units, 4. Evolution of Computers, 5. Von Neumann Model & Harvard / CISC vs RISC, 6. Performance Measure of Computer Architecture, 7. Architecture of 8086 Family, 8. Instruction Set, 9. Addressing Modes. Course Outcomes (COs): CO1 (8086 architecture & assembly programming), CO2 (Control unit design & ILP), CO3 (Arithmetic algorithms & digital logic), CO4 (Memory hierarchy & cache), CO5 (I/O mechanisms & interfacing), CO6 (8051 microcontroller architecture & C programming).

1. Introduction to Computer Organization and Architecture

A Computer is a combination of integrated hardware and software resources that work together to provide diverse functionalities to the user.

1.1 Definitions & Conceptual Differentiation

Computer Architecture:

• Functional Behavior: Deals with the functional behavior and structure of a computer system as seen by the user/programmer.

• Design Specification: Serves as the functional description of requirements and design implementation for the various parts of a computer.

• Design Sequence: Comes BEFORE computer organization during the computer design process.

• Programmer Visibility: Refers to the system attributes that have a direct impact on the logical execution of a program (e.g., Instruction Set, data representation bit-widths, I/O mechanisms, addressing techniques).

Computer Organization:

• Structural Relationship: Deals with the structural relationships and physical interconnections between operational hardware units.

• Implementation: Operational attributes are linked together and contribute to physically realize the architectural specifications.

• Sequence: Decided AFTER the architecture is fixed.

• Transparency: Encompasses low-level hardware design details that escape the programmer's detection (e.g., control signals, memory technologies, peripheral interfacing, adders).

📌 SLIDE CORE ANALOGIES & FUNDAMENTAL PRINCIPLES Core Conceptual Rule: 'Designing is the attribute of Architecture, and utilisation happens to be the attribute of Organisation.' The Iron Man Metaphor (Slide 6): The internal blueprint/skeleton is the Architecture, while the physical armored suit and motorized actuators represent the Organization. The Half Adder Analogy (Slide 7): A 2-bit addition specification (Truth table: 0+0=00, 0+1=01, 1+0=01, 1+1=10) is the Architecture. Constructing it physically using NAND gates is the Organization.

1.2 Comprehensive Comparison: Architecture vs. Organization

The comprehensive 13-point comparison table presented in the lecture slides:

| S.No. | Computer Architecture | Computer Organization |
| --- | --- | --- |
| 1 | Architecture describes WHAT the computer does. | Organization describes HOW it does it. |
| 2 | Deals with the functional behavior of computer systems. | Deals with structural relationships and operational linkages. |
| 3 | Deals with high-level design issues. | Deals with low-level design issues. |
| 4 | Architecture indicates its hardware capabilities. | Organization indicates its operational performance. |
| 5 | Viewed by programmer as a series of instructions, addressing modes, and registers. | The physical implementation of the architecture is called organization. |
| 6 | For designing a computer, its architecture is fixed first. | Organization is decided after its architecture is specified. |
| 7 | Also called Instruction Set Architecture (ISA). | Frequently called Microarchitecture. |
| 8 | Comprises logical functions (instruction sets, registers, data types, addressing modes). | Consists of physical units (circuit designs, peripherals, adders, buses). |
| 9 | Architectural categories: Von Neumann, Harvard, ISA, Micro-architecture, System Design. | CPU organization classified by address fields: (1) Single Accumulator, (2) General Registers, (3) Stack Organization. |
| 10 | Makes the computer's functional hardware visible. | Offers details on how well the computer performs. |
| 11 | Coordinates the hardware and software of the system. | Handles physical segments of the system network. |
| 12 | The software developer/compiler writer is aware of it. | It escapes the software programmer's detection. |
| 13 | Examples: Intel & AMD x86, Sun SPARC, Apple/IBM/Motorola PowerPC. | Examples: Interfacing of computer and peripherals, memory technologies, control signals. |

1.3 Classification of Architecture & CPU Organization

Computer Architecture is subdivided into two primary branches (Slide 23):

1. Instruction Set Architecture (ISA): The instruction repertoire, register set, and programming interface.

2. Hardware System Architecture (HSA): The physical hardware components and subsystem interconnects (adders, registers, datapath).

CPU Organization is classified into three categories based on the number of address fields (Slide 22, 24):

1. Single Accumulator Organization: Uses an implicit Accumulator register for all operations (e.g., LDI 02H, ADI 03H, STA X).

2. General Register Organization: Uses multiple general-purpose registers (e.g., MOV R1, 02H; MOV R2, 03H; ADD R1, R2; STORE X, R1 or 3-address: ADD X, 02H, 03H).

3. Stack Organization: Uses Zero-Address instructions operating on a LIFO stack (e.g., PUSH 02H, PUSH 03H, ADD, POP X).

1.4 Hierarchy of a Computer System

The computer system hierarchy bridges software applications down to physical silicon (Slide 26):

[HIGH LEVEL / SOFTWARE] Application Program ¦ Application Design ¦ System Design <--- [COMPUTER ARCHITECTURE] ¦ Computer Design <--- [COMPUTER ARCHITECTURE / ORGANIZATION] ¦ Logic Design <--- [COMPUTER ORGANIZATION] ¦ Circuit Design <--- [COMPUTER ORGANIZATION] ¦ Computer Component [LOW LEVEL / HARDWARE]

1.5 Structure and Function of Computers

• Structure: The way in which components relate to each other.

• Function: The operation of individual components as part of the structure.

The 4 universal computer functions (Slide 27-28):

1. Data Processing: Processing data in wide variety of forms and ranges.

2. Data Storage: Storing data either temporarily (RAM/registers) or permanently (disks).

3. Data Movement: Moving data between the computer and the outside world.

4. Control Mechanism: Orchestrating all resources via the Control Mechanism.

Top-Level Structural Decomposition (Slide 29):

• Top Level (Computer): CPU, Main Memory, I/O, interconnected via the System Bus.

• CPU Level: Registers, Arithmetic & Logic Unit (ALU), Control Unit, Internal Bus.

• Control Unit Level: Sequencing Logic, Control Unit Registers and Decoders, Control Memory.

2. Basic Organization of Computer and Functional Units

A computer is not a single unit but consists of multiple cooperating functional units (Slide 30-35): (1) Input Unit, (2) Storage/Memory Unit, (3) Central Processing Unit (ALU & Control Unit), and (4) Output Unit.

+-------------------------------------------------------------------------+ ¦ BLOCK DIAGRAM OF COMPUTER (Slide 30) ¦ ¦ ¦ ¦ +---------------------------------+ ¦ ¦ ¦ STORAGE UNIT ¦ ¦ ¦ ¦ +---------------------------+ ¦ ¦ ¦ ¦ ¦ Secondary Storage ¦ ¦ ¦ ¦ ¦ +-------------^-------------+ ¦ ¦ ¦ ¦ ¦ (Data) ¦ ¦ ¦ ¦ +-------------v-------------+ ¦ ¦ ¦ ¦ ¦ Primary Storage ¦ ¦ ¦ ¦ ¦ +---------------------------+ ¦ ¦ ¦ +----------------¦----------------+ ¦ ¦ ¦ ¦ ¦ +------------+ ¦ (Data) +------------+ ¦ ¦ Data ¦ INPUT ¦------------------+------------------>¦ OUTPUT ¦ ¦ Info ¦ ---->¦ UNIT ¦...... ¦ UNIT ¦ ¦----> ¦ +------------+ : +-----^------+ ¦ ¦ : (Control) ¦ ¦ ¦ v : (Ctrl) ¦ ¦ +---------------------------------+ : ¦ ¦ ¦ CENTRAL PROCESSING (CPU) ¦ : ¦ ¦ ¦ +---------------------------+ ¦ : ¦ ¦ ¦ ¦ Control Unit (CU) ¦..¦........: ¦ ¦ ¦ +-------------^-------------+ ¦ ¦ ¦ ¦ : (Control) ¦ ¦ ¦ ¦ +-------------v-------------+ ¦ ¦ ¦ ¦ ¦ Arithmetic & Logic (ALU) ¦ ¦ ¦ ¦ ¦ +---------------------------+ ¦ ¦ ¦ +---------------------------------+ ¦ ¦ Legend: ------> Data Flow ........> Control Flow ¦ +-------------------------------------------------------------------------+

2.1 Block-Level Description of Functional Units

1. Central Processing Unit (CPU) - 'The Brain':

Controls the operation of the computer and performs its data processing functions (often simply referred to as processor). Main parts include:

• Instruction Register (IR): Holds the instruction to be executed.

• Decoder: Decodes (converts to machine-level language) the instruction and sends signals to the ALU.

• Arithmetic Logic Unit (ALU): Contains necessary circuits to perform arithmetic, logical, memory, register, and program sequencing operations.

• Registers (Slide 33): Holds intermediate results obtained during program processing. Registers are used rather than RAM because accessing registers is almost 10 times faster than accessing RAM.

2. Memory Unit (Slide 34):

The microprocessor interacts with two fundamental types of memory:

• RAM (Random Access Memory): Volatile memory that gets erased when power is switched off. Stores all executing data and instructions during runtime.

• ROM (Read Only Memory): Non-volatile memory whose data remains intact even after power is switched off. The microprocessor can read from it at any time but cannot write to it. Pre-programmed with essential start-up data like the booting sequence by the manufacturer.

Summary distinction: RAM is used for storage of active data; ROM is used for storage of permanent programs (especially the start-up program executed when the system is powered on).

3. Input / Output (I/O) Devices (Slide 35):

• Function: Allow the computer to communicate with the outside world.

• Input Devices: Supply alphanumeric, image, audio, and video data to the computer for processing. Examples: Keyboard, mouse, scanner, microphone, camera.

• Output Devices: Display processed information to the user. Examples: Monitor, printer.

• Connection: Connected to the computer subsystems through the System Bus.

4. System Bus (Slide 54):

Connection lines used to connect the internal parts of the microprocessor chip are called a Bus. There are three types of buses:

• Data Bus: Lines that carry data to and from memory/IO. Bidirectional bus with width equal to the word length of the microprocessor.

• Address Bus: Unidirectional lines responsible for carrying the address of a memory location or I/O port from the CPU to memory/IO.

• Control Bus: Bidirectional lines carrying control and synchronization signals (Clock, Interrupt, Ready). Ready signal denotes a device is ready for processing; Interrupt signal indicates to a device to interrupt its process.

3. Evolution of Computers

Computers have evolved through distinct technological phases from room-sized vacuum tube mainframes to ultra-compact AI-driven microchips (Slide 55-67).

3.1 Basic Hardware Terms

• Vacuum Tube: Fragile glass device that controls the flow of electrons in a vacuum. Used as switches and amplifiers in first-generation computers.

• Transistor: Semiconductor device invented at Bell Labs that controls electricity flow, acting as an amplifier or electronic switch.

• Integrated Circuit (IC): Silicon chip containing thousands to billions of microscopic transistors, resistors, and capacitors on a single die.

3.2 Phases of Computer Generations (Slide 56-66)

| Generation | Time Period | Key Electronic Component | Memory & I/O | Programming Language | Key Examples |
| --- | --- | --- | --- | --- | --- |
| First Generation | 1940s – 1950s | Vacuum Tubes (18,000+ tubes in ENIAC), fragile glass. | Magnetic drums & tapes. Paper tape, punched cards. | Machine language & primitive assembly. No OS. | ENIAC, EDVAC (Von Neumann), UNIVAC I (1952), IBM 650, IBM 701. |
| Second Generation | 1950s – 1960s | Transistors (Bell Labs). Smaller size, lower power, reduced heat. | Magnetic core memory, magnetic tape/disk. Punched cards. | High-level languages: FORTRAN (1956), ALGOL (1958), COBOL (1959). | PDP-8, IBM 1400 series, IBM 7090/7094, UNIVAC 1107, CDC 3600. |
| Third Generation | 1960s – 1970s | Integrated Circuits (ICs) / Silicon semiconductor chips. | Large magnetic core, semiconductor memory, magnetic disks. Keyboard, monitor. | High-level languages like BASIC. Operating Systems (Time-sharing & multiprogramming). | IBM 360, IBM 370, PDP-11, NCR 395, B6500, UNIVAC 1108. |
| Fourth Generation | 1970s – Present | LSI & VLSI Microprocessors (Intel 4004 in 1971, 8086). All CPU/mem/IO on one chip. | Semiconductor RAM/ROM, Floppy, Hard disks, SSDs. Pointing devices, optical scan. | High-level OOP languages (C, C++, Java, Python). Virtual memory, networking, PCs. | Intel 4004, 8086, IBM PC, Apple II, Macintosh, Altair 8800, STAR 1000. |
| Fifth Generation | Present – Future | Ultra Large-Scale Integration (ULSI), Artificial Intelligence, Parallel Processing. | High-speed 3D memory, cloud storage. Touchscreen, trackpad, pen, speech input. | Natural language understanding (human language), AI models. | AI Supercomputers, Neural Accelerators, Modern laptops/mobile devices (started Japan MITI 1982). |

3.3 Transistor Density Growth Milestones (Slide 67)

Historical scaling of transistor counts on a single silicon die:

• 1950s: 1 Silicon Transistor

• 1960s: 16 Transistors (TTL Quad Gate)

• 1970s: 4,500 Transistors (8-bit Microprocessor)

• 1980s: 275,000 Transistors (32-bit Microprocessor)

• 1990s: 3,100,000 Transistors (32-bit Microprocessor)

• 2000s: 592,000,000 Transistors (64-bit Microprocessor)

• 2010s+: 8,000,000,000+ Transistors (3072-Core GPU / Modern Processors)

4. The Von Neumann Model, Harvard Architecture, and CISC vs. RISC

The Von Neumann Architecture is a theoretical computer design proposed by mathematician John von Neumann in 1945 based on the Stored-Program Concept, serving as the foundation of almost all modern general-purpose computers (Slide 36-47).

4.1 Key Components & Working of Von Neumann Model (Slide 38-40)

1. Central Processing Unit (CPU): Comprises Control Unit (fetches, decodes, executes instructions) and Arithmetic Logic Unit (handles math and logic).

2. Memory Unit (Stored-Program Concept): Stores both data and instructions in a single, unified addressable memory space organized as a linear sequence of addressable cells.

3. Input/Output (I/O) System: Handles interaction with external devices like keyboards, monitors, and printers.

4. System Bus: Transfers data, addresses, and control signals between CPU, memory, and I/O devices (Data Bus, Address Bus, Control Bus).

How the Von Neumann Machine Works (The 4-Step Cycle):

• Fetch: The CPU retrieves an instruction from memory using the Program Counter (PC).

• Decode: The Control Unit interprets and decodes the fetched instruction.

• Execute: The ALU performs the required arithmetic or logical operation.

• Store: The result is written back to memory or register if needed.

4.2 Advantages & Limitations of Von Neumann Architecture (Slide 41-42)

• Simplicity: Straightforward design makes it easier to build and understand.

• Flexibility: Can execute various programs simply by changing the stored instructions in memory.

• Efficiency & Easy Access: Sharing the same memory for instructions and data reduces hardware complexity and cost.

• Von Neumann Bottleneck (Limitation): The CPU and memory share the same bus, causing delays because only one piece of data or instruction can be accessed at a time. This limits overall speed.

• Serial Execution Limitation: The CPU can only execute one instruction at a time.

• Shared Memory Risk: Because instructions and data reside in the same memory, instructions can be accidentally modified by data, leading to errors or security risks.

4.3 Harvard Architecture & The Modern Hybrid Approach (Slide 43-47)

In the Harvard Architecture, program instructions and data are stored in completely separate physical memories with their own dedicated buses.

• Advantage: Faster execution because the CPU can fetch an instruction and access data concurrently in a single clock cycle, completely eliminating the bottleneck.

• Limitation: More complex and expensive; unused instruction memory space cannot be reallocated for data.

• Use Cases: Microcontrollers (e.g., 8051, AVR, PIC), Digital Signal Processors (DSP), audio/video processing, real-time embedded systems.

Modern Hybrid Approach (Slide 46):

Most modern desktop and mobile CPUs use a combination of both: at a high level, they use a Von Neumann structure to fetch data from main RAM, but internally, the CPU divides its Level 1 (L1) Cache into separate instruction and data caches, effectively utilizing a Modified Harvard Architecture.

| Comparison Feature | Von Neumann Architecture | Harvard Architecture |
| --- | --- | --- |
| Origin & Basis | Ancient architecture based on stored-program concept. | Modern architecture based on Harvard Mark I relay model. |
| Memory Address Space | Same physical memory address used for instructions and data. | Separate physical memory addresses for instructions and data. |
| Bus System | Common shared bus for data and instruction transfer. | Separate independent buses for data and instruction transfer. |
| Execution Speed | Two clock cycles required to execute single instruction. | An instruction can be executed in a single clock cycle. |
| Cost & Complexity | Cheaper in cost and simpler in design. | More costly and complex to route. |
| Memory Concurrency | CPU cannot access instructions and read/write data at the same time. | CPU can access instructions and read/write data at the same time. |
| Primary Usage | Personal computers and general-purpose computers. | Microcontrollers, DSPs, and real-time signal processing. |

4.4 CISC vs. RISC Architecture (Slide 48-51)

Microprocessors are broadly divided into Complex Instruction Set Computers (CISC) and Reduced Instruction Set Computers (RISC):

• CISC (Complex Instruction Set Computer): Basic idea: Give programmers powerful instructions so fewer instructions are needed to complete the work (e.g., VAX has ~325 instructions, x86, Motorola 68K).

• RISC (Reduced Instruction Set Computer): Basic idea: Compose simple single-cycle instructions to get complex results (e.g., MIPS has ~100 instructions, ARM, RISC-V, SPARC).

| Attribute / Parameter | CISC Architecture | RISC Architecture |
| --- | --- | --- |
| Full Form | Complex Instruction Set Computing | Reduced Instruction Set Computing |
| Design Emphasis | Emphasizes hardware complexity. | Emphasizes software / compiler optimization. |
| Control Unit Type | Functions both on Microprogrammed and Hardwired CU. | Functions ONLY on Hardwired Control Unit. |
| Transistor Usage | Transistors used for storing complex instruction microcode. | Transistors used for large register files. |
| Register Count | Registers are used in less quantity. | Many general-purpose registers are used. |
| Instruction Size | Variable instruction size (1 byte to 15+ bytes). | Fixed instruction size (typically 32-bit). |
| Memory Operations | Can perform REG-to-REG, REG-to-MEM, and MEM-to-MEM. | Load/Store architecture; Arithmetic is strictly REG-to-REG. |
| Code / Binary Size | Size of the compiled code is small/compact. | Size of the compiled code is larger. |
| Clock Cycles per Instruction | More than one clock cycle required per instruction (multi-cycle). | Instructions executed in a single clock cycle. |
| Addressing Modes | Addressing modes are complex and large in number. | Addressing modes are simple and limited. |
| Power Consumption | Consumes more electrical power. | Consumes less power (ideal for mobile/battery devices). |
| RAM Usage | Less or efficient usage for RAM storage. | Requires heavy usage of RAM storage. |
| Pipelining Dependency | Less dependency / harder to pipeline. | Highly dependent on pipelining efficiency. |
| Procedure Calls | Uses Stack for return addresses and procedure arguments. | Uses Registers for return addresses & arguments (avoids memory). |
| Cache Architecture | Unified cache used for data and instructions. | Split instruction cache and data cache. |
| Examples | x86, Motorola 68K, PDP-11, VAX, Personal Computers. | ARM, RISC-V, MIPS, SPARC, Atmel AVR, PowerPC, DEC Alpha. |

5. Performance Measure of Computer Architecture

In computer organization, performance refers to the speed and efficiency at which a computer system executes tasks and processes data (Slide 68-86).

5.1 Subsystem Performance & The 3 Core Metrics

Performance is measured across multiple subsystems:

• CPU Performance (Scientific/Vector/Business): Instructions per Second (MIPS/GIPS).

• Graphics Performance: Rendering rate in Pixels per second / Frames per second.

• I/O Performance: Transactions Per Second (TPS).

• Network / Internet Performance: Bandwidth utilization in Mbps or Gbps.

The 3 Fundamental System Performance Metrics (Slide 70-73):

1. Response Time (Latency): The total elapsed time from start to completion of a task. Includes CPU execution time, OS overhead, I/O wait times, and memory access delays. Lower response time is crucial for interactive applications.

2. CPU Execution Time: The time the CPU actually spends computing on a given task, excluding any waiting time for I/O or other processes.

3. Throughput: The total amount of work completed in a given period of time (tasks/sec or instructions/sec). Important for servers handling database transactions or web requests.

Fundamental Relationships (Slide 74):

Total Execution Time = CPU Time + I/O Time + Other Delays (Queuing, OS overhead) Performance = 1 / Execution Time

5.2 The Iron Law of Processor Performance (Slide 74-75)

CPU Execution Time ($T_{\text{CPU}}$) is calculated using the Iron Law:

CPU Time (T_CPU) = Number of Instructions (N) x Average CPI x Clock Cycle Time (T_clk) = (N x CPI) / Clock Frequency (f) Where: N = Total Instruction Count in the program (influenced by Source Code, Compiler, ISA) CPI = Average Clock Cycles per Instruction (influenced by ISA, Microarchitecture) T_clk = Time per clock cycle = 1 / Clock Frequency (influenced by Microarchitecture, Hardware Tech)

5.3 Slide Worked Examples & Numerical Calculations

🧮 SLIDE WORKED EXAMPLE: Example 1 (Program ABCD Execution Time) Problem: A program ABCD has 15,000 instructions (N = 15,000) is executed on a system whose clock frequency is 3.3 GHz and the design facilitates average Cycles per Instruction (CPI) of 12. Calculate the CPU time utilized to execute Program ABCD. (Slide 76-77) Step-by-Step Solution / Calculation: • Given: N = 15,000 instructions, CPI = 12, Clock Frequency f = 3.3 GHz = 3.3 x 10^9 Hz. • Calculate Clock Cycle Time: T_clk = 1 / (3.3 x 10^9) = 0.303 x 10^-9 seconds (approx 0.3 x 10^-9 s). • Apply CPU Time Formula: T_CPU = N x CPI x T_clk • Calculation: T_CPU = 15,000 x 12 x (0.3 x 10^-9 s) = 54,000 x 10^-9 seconds = 54 x 10^-6 seconds = 54 microseconds (54 μs).

🧮 SLIDE WORKED EXAMPLE: Examples 2, 3 & 4 (Scaling & Superscalar Comparison) Problem: Compare CPU Execution Times for Program P (1 Billion instructions, N = 10^9) across 3 architectural scenarios: (a) Baseline: CPI = 1, Clock = 1 GHz; (b) Frequency Scaling: CPI = 1, Clock = 4 GHz; (c) Superscalar: Processes 10 instructions in one cycle (CPI = 0.10), Clock = 4 GHz. (Slides 78-80) Step-by-Step Solution / Calculation: • Scenario (a) Baseline (1 GHz, CPI = 1): T_CPU = 10^9 instructions x 1 cycle/inst x 1 ns = 1.000 second. • Scenario (b) Frequency Scaling (4 GHz, CPI = 1): T_CPU = 10^9 instructions x 1 cycle/inst x (1/4 ns) = 0.250 second (4X Faster). • Scenario (c) Superscalar Execution (4 GHz, 10 instrs/cycle -> CPI = 0.10): T_CPU = 10^9 instructions x 0.10 cycle/inst x (1/4 ns) = 0.025 second (40X Faster!).

🧮 SLIDE WORKED EXAMPLE: Role of Compiler / Programmer Optimization Problem: Analyze the role of compiler optimization: If an optimized compiler reduces the instruction count of Program P from 1 Billion to 1 Million instructions (N = 10^6), running on a 4 GHz processor with CPI = 1. (Slide 81) Step-by-Step Solution / Calculation: • Calculation: T_CPU = 10^6 instructions x 1 cycle/inst x (1/4 ns) = 0.00025 second. • Performance Impact: 4,000X faster than the 1-second baseline!

🧮 SLIDE WORKED EXAMPLE: 'A Bit Deeper' Multi-Class CPI Calculation Problem: Program P has 10 Billion total instructions executed on a 4 GHz processor: • 2 Billion branches (CPI = 4) • 3 Billion Loads (CPI = 2) • 1 Billion Stores (CPI = 3) • Rest 4 Billion Arithmetic instructions (CPI = 1) Calculate the overall CPU execution time. (Slide 82) Step-by-Step Solution / Calculation: • Step 1: Calculate Total Clock Cycles required for each instruction class: • Branch Cycles = 2 x 10^9 x 4 = 8 x 10^9 cycles • Load Cycles = 3 x 10^9 x 2 = 6 x 10^9 cycles • Store Cycles = 1 x 10^9 x 3 = 3 x 10^9 cycles • Arithmetic Cycles = 4 x 10^9 x 1 = 4 x 10^9 cycles • Step 2: Total Clock Cycles = 8 + 6 + 3 + 4 = 21 Billion cycles (21 x 10^9 cycles). • Step 3: Calculate CPU Execution Time (Clock = 4 GHz): T_CPU = (21 x 10^9 cycles) / (4 x 10^9 cycles/sec) = 5.25 seconds. • Step 4: Average System CPI = Total Cycles / Total Instructions = 21 x 10^9 / 10 x 10^9 = 2.1 CPI.

🧮 SLIDE WORKED EXAMPLE: 'Which One?' Architectural Trade-Off Problem: Compare two competing processor designs assuming identical compiler and ISA: Processor IMTEL (CPI = 2, Clock Rate = 2 GHz) vs Processor AND (CPI = 1, Clock Rate = 1 GHz). Which one is faster? (Slide 83) Step-by-Step Solution / Calculation: • Processor IMTEL Time per instruction = CPI x T_clk = 2 x (1 / 2 GHz) = 2 x 0.5 ns = 1.0 ns per instruction. • Processor AND Time per instruction = CPI x T_clk = 1 x (1 / 1 GHz) = 1 x 1.0 ns = 1.0 ns per instruction. • Conclusion: Both processors have identical execution performance (1 ns/instruction)!

5.4 Performance-Enhancing Paradigms (Slide 85-86)

1. Pipelining: Overlaps instruction execution stages. While instruction 1 is executed, instruction 2 is decoded, and instruction 3 is fetched.

2. Superscalar Execution: Incorporates multiple ALUs/execution units to execute more than one instruction simultaneously during a single clock cycle (IPC > 1).

3. Out-of-Order Execution (OoO): Executes instructions dynamically based on data availability rather than strict program order, preventing execution pipeline stalls.

6. Architecture of 8086 Family

The Intel 8086 is a 16-bit microprocessor introduced in 1978, containing ~29,000 transistors in a 40-pin Dual In-line Package (DIP) operating on +5V (Slide 87-130).

6.1 Core Hardware Features of 8086 (Slide 89-90)

1. Word Length: 16-bit microprocessor; ALU and internal registers operate on 16-bit binary words.

2. Packaging & Complexity: 40-pin Dual In-line Package (DIP) containing ~29,000 transistors.

3. Memory Addressability: 20-bit address bus can access up to 2^20 = 1,048,576 bytes (1 MB) of memory locations (range: 00000H to FFFFFH). Memory is segmented into 16 sections of 64 KB each.

4. Data Bus: 16-bit data bus; reads or writes data either 16 bits or 8 bits at a time.

5. I/O Ports: Supports up to 64K (65,536) independent I/O ports.

6. Registers: Provides 14 internal registers, all 16 bits wide.

7. Operating Frequency & Clock: Operates at 5 MHz (standard), 8 MHz (8086-2), and 10 MHz (8086-1). Requires single-phase clock with 33% duty cycle.

8. Bus Multiplexing: Time-multiplexed address/data bus (AD0–AD15) and address/status bus (A16/S3–A19/S6).

9. 6-Byte Prefetch Queue: Prefetches up to 6 instruction bytes in a FIFO queue to speed up execution (primitive pipelining).

10. Operating Modes: Operates in two hardware modes: Minimum Mode (single processor) and Maximum Mode (multiprocessor).

6.2 Internal Architecture: BIU and EU (Slide 91-110)

The 8086 CPU is split into two independent functional units:

A. Bus Interface Unit (BIU) (Slide 93-96):

Interfaces the 8086 to external memory and I/O devices via the System Bus. Responsibilities:

Generates 20-bit physical addresses using the internal Address Adder ($PA = \text{Segment} \times 10\text{H} + \text{Offset}$).

Fetches instructions from memory and maintains the 6-byte prefetch instruction queue.

Transfers data to and from memory and I/O ports.

Contains: 4 Segment Registers (CS, DS, SS, ES), Instruction Pointer (IP), 6-byte Prefetch Queue, and 20-bit Address Adder.

Prefetch Mechanism: While the EU is executing an instruction that does not require the buses, the BIU prefetches up to 6 bytes into the FIFO queue. If a JMP or CALL occurs, the queue is dumped and reloaded from the new target address.

B. Execution Unit (EU) (Slide 106-110):

Responsible for decoding and executing instructions. Responsibilities:

Extracts instructions from the top of the BIU queue.

Decodes instructions using the Instruction Decoder.

16-bit ALU performs arithmetic (add, subtract, increment, decrement) and logical operations (AND, OR, XOR, complement, shift).

Tests and updates status and control flags in the Flag Register.

If the queue is empty, the EU waits for the next instruction byte to be fetched.

6.3 Register Organization of 8086 (Slide 111-125)

The 8086 has 14 programmer-accessible 16-bit registers classified into 4 functional groups:

| Register Group | Registers Included | Bit Width | Primary Dedicated Purpose |
| --- | --- | --- | --- |
| General Purpose Registers | AX (AH/AL), BX (BH/BL), CX (CH/CL), DX (DH/DL) | 16-bit (or 8-bit pairs) | Arithmetic, logic, data storage, port I/O, counters. |
| Segment Registers | CS (Code), DS (Data), SS (Stack), ES (Extra) | 16-bit | Base addresses of 64 KB memory segments. |
| Pointer & Index Registers | SP (Stack Ptr), BP (Base Ptr), SI (Source Idx), DI (Dest Idx) | 16-bit | Offset storage for stack frames, data, and strings. |
| Program Status / Control | IP (Instruction Pointer), Flags Register | 16-bit | Next instruction offset (IP) and CPU status condition codes. |

Detailed Function of General Purpose Registers (Slide 113-118):

• AX: Accumulator (AH/AL). Word/byte multiply, divide, I/O operations, string manipulation, decimal arithmetic. (Example: AX = 1234H -> AH = 12H, AL = 34H).

• BX: Base Register (BH/BL). Holds data pointer for based, based-indexed, and register indirect addressing.

• CX: Count Register (CH/CL). Default counter for loop instructions, string repeats (REP), and variable bit shifts/rotates (CL).

• DX: Data Register (DH/DL). Holds 16-bit I/O port address for IN/OUT instructions; holds high-order 16 bits of 32-bit multiply/divide results (DX:AX).

Pointer & Index Registers (Slide 121-123):

• SP (Stack Pointer): Points to the top of the stack (LIFO). Decremented by PUSH, incremented by POP.

• BP (Base Pointer): Points to data/parameters inside the stack segment (used by subroutines to locate passed parameters).

• SI (Source Index): Source index pointer for indexed addressing and string source data (default DS:SI).

• DI (Destination Index): Destination index pointer for indexed addressing and string destination data (default ES:DI).

Operand Registers (Slide 125): Temporary registers not visible to the programmer, used internally by the CPU to store intermediate values during operations (e.g., during `XCHG BX, CX`).

6.4 The 16-Bit Flag Register (Slide 126-129)

Contains 9 active 1-bit flags (6 Status flags and 3 Control flags); 7 bits are unused/undefined:

Bit: 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0 Flag: X X X X OF DF IF TF SF ZF X AF X PF X CF Control Flags: DF (Direction), IF (Interrupt Enable), TF (Trap/Single Step) Status Flags: CF (Carry), PF (Parity), AF (Auxiliary Carry), ZF (Zero), SF (Sign), OF (Overflow)

| Flag Name | Bit Position | Type | Conditions for Setting (Flag = 1) |
| --- | --- | --- | --- |
| CF (Carry Flag) | D0 | Status | Set if addition generates a carry out of MSB or subtraction generates a borrow. |
| PF (Parity Flag) | D2 | Status | Set if the lowest 8 bits of the result contain an EVEN number of 1s. |
| AF (Auxiliary Carry) | D4 | Status | Set if carry generated from lower nibble (D3) to higher nibble (D4) in 8-bit arithmetic. |
| ZF (Zero Flag) | D6 | Status | Set if the result of an operation is exactly 00H / 0000H. |
| SF (Sign Flag) | D7 | Status | Set if MSB of result is 1 (negative number). Cleared if MSB is 0 (positive). |
| TF (Trap Flag) | D8 | Control | When set, enables single-step execution mode for debugging (INT 1 after each instruction). |
| IF (Interrupt Flag) | D9 | Control | When set (STI), enables INTR hardware maskable interrupts; when 0 (CLI), disables INTR. |
| DF (Direction Flag) | D10 | Control | When 0 (CLD), auto-increments SI/DI; when 1 (STD), auto-decrements SI/DI in string operations. |
| OF (Overflow Flag) | D11 | Status | Set if signed arithmetic result exceeds capacity. Calculated as Carry_into_MSB XOR Carry_out_of_MSB (OF = C7 XOR C6). |

🧮 SLIDE WORKED EXAMPLE: Slide 129 Flag Evaluation (7FH + FEH) Problem: Determine the status of all condition flags when the 8086 ALU performs the addition: 7FH + FEH. (Slide 129) Step-by-Step Solution / Calculation: • Binary representation: 7FH = 0 1 1 1 1 1 1 1 + FEH = 1 1 1 1 1 1 1 0 ------------------------- Sum = 1 0 1 1 1 1 1 0 1 (Hex: 17DH -> 8-bit result is 7DH with Carry = 1) • Carry Analysis: • Carry out of bit D3 into bit D4 = 1 ==> AF = 1 • Carry into MSB (bit D7) = 1 • Carry out of MSB (bit D7) = 1 ==> CF = 1 • Flag Evaluation: • CF = 1 (Carry out of MSB = 1) • PF = 1 (Total number of 1s in 7DH = 01111101_2 has six 1s, which is EVEN) • AF = 1 (Carry generated from D3 to D4) • ZF = 0 (Result 7DH != 00H) • SF = 0 (MSB bit D7 is 0, indicating positive) • OF = 0 (Carry into MSB = Carry out of MSB = 1; OF = 1 XOR 1 = 0, result is correct signed value).

6.5 Pin Diagram & Detailed Signal Descriptions (Slide 130, 188-208)

The 8086 is a 40-pin DIP chip. Its signals are categorized into: (1) Common signals (both modes), (2) Minimum mode signals ($MN/\overline{MX}=1$), and (3) Maximum mode signals ($MN/\overline{MX}=0$).

Common Signals (Both Modes):

• VSS (GND): Pins 1 & 20. Ground reference.

• VCC: Pin 40. +5V DC power supply input.

• AD15–AD0: Pins 2–16, 39. Time-multiplexed memory/IO address (during T1 state) and data bus (during T2, T3, TW, T4 states). Tristated during hold acknowledge.

• A19/S6–A16/S3: Pins 35–38. Time-multiplexed upper 4 address lines (during T1) and status lines S3–S6 (during T2–T4). S6 is always low. S5 displays Interrupt Enable Flag status. S4 & S3 indicate active segment register:

| S4 | S3 | Indication (Active Segment Register) |
| --- | --- | --- |
| 0 | 0 | Extra Segment (ES) / Alternate Data |
| 0 | 1 | Stack Segment (SS) |
| 1 | 0 | Code Segment (CS) or None |
| 1 | 1 | Data Segment (DS) |

• BHE'/S7: Pin 34. Bus High Enable / Status S7. Identifies existence and byte width on the bus (00 = 16-bit word, 01/10 = 8-bit byte on high bus, 11 = idle).

• MN/MX': Pin 33. Tied to +5V for Minimum Mode (single CPU); tied to GND for Maximum Mode (multiprocessor).

• RD': Pin 32. Active-low Read strobe indicating CPU is reading memory or I/O.

• CLK: Pin 19. System clock input (provides fundamental internal timing).

• NMI: Pin 17. Non-Maskable Interrupt. Edge-triggered high-priority interrupt vector.

• INTR: Pin 18. Level-triggered maskable interrupt request input.

• READY: Pin 22. Acknowledgement signal from slow memory/IO devices; pulling low forces CPU wait states (TW).

• RESET: Pin 21. Active-high reset input; held high for at least 4 cycles to restart execution at address FFFF0H.

• TEST': Pin 23. Tested by WAIT instruction; if high, CPU enters idle wait state until TEST' goes low.

Minimum Mode Signals (Pins 24–31 when MN/MX' = 1):

• INTA': Pin 24. Interrupt Acknowledge output generated in response to INTR.

• ALE: Pin 25. Address Latch Enable; positive pulse indicating address is valid on multiplexed bus AD0-AD15 (demultiplexes address).

• DEN': Pin 26. Data Enable; active-low strobe to enable external bidirectional transceivers (8286).

• DT/R': Pin 27. Data Transmit/Receive; high indicates CPU transmitting data, low indicates CPU receiving data.

• M/IO': Pin 28. Memory/IO select; high indicates Memory operation, low indicates I/O operation.

• WR': Pin 29. Active-low Write strobe indicating CPU is writing data to memory or I/O.

• HLDA: Pin 30. Hold Acknowledge; indicates CPU has received HOLD request and released system buses.

• HOLD: Pin 31. Hold Request; external master requests access to address and data buses.

Maximum Mode Signals (Pins 24–31 when MN/MX' = 0):

• S0', S1', S2': Pins 26–28. Active-low processor status lines decoded by external 8288 Bus Controller:

| S2' | S1' | S0' | Processor Bus Cycle Status (Decoded by 8288) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Interrupt Acknowledge (INTA) |
| 0 | 0 | 1 | Read I/O Port |
| 0 | 1 | 0 | Write I/O Port |
| 0 | 1 | 1 | Halt |
| 1 | 0 | 0 | Code Access / Instruction Fetch |
| 1 | 0 | 1 | Read Memory |
| 1 | 1 | 0 | Write Memory |
| 1 | 1 | 1 | Passive / Inactive State |

• QS0, QS1: Pins 24 & 25. Queue Status lines indicating BIU instruction queue operations (00 = No op, 01 = First byte from queue, 10 = Empty queue, 11 = Subsequent byte from queue).

• LOCK': Pin 29. Bus lock output; asserted during atomic instructions to prevent other bus masters from taking control.

• RQ'/GT0', RQ'/GT1': Pins 30 & 31. Bidirectional Request/Grant lines for multiprocessor bus arbitration.

7. Instruction Set of 8086

The 8086 instruction set defines the machine operations executed by the processor. Each instruction consists of an Opcode (operation mnemonic) and Operands (destination and source) (Slide 131-173).

7.1 Data Movement Instructions (Slide 133-141)

• MOV Destination, Source: Transfers data from Source to Destination. Source can be register, memory, or immediate data; Destination can be register or memory. (Both cannot be memory at the same time!).

• PUSH Operand: Pushes operand into top of stack; decrements SP by 2. (Example: `PUSH BX`).

• POP Destination: Pops operand from stack top into Destination; increments SP by 2. (Example: `POP AX`, `POP DS`).

• XCHG Des, Src: Swaps contents of two registers or a register and memory. Cannot swap two memory locations directly. (Example: `XCHG BX, AX`).

• LEA (Load Effective Address): Loads the 16-bit offset memory address into a register. (Example: `LEA BX, [SI + 04H]`).

Worked Visual Assembly Traces from Slides:

• PUSH BX Trace: If BX = 10ABH and SP = 5002H: SP decrements to 5001H and stores 10H (BH); SP decrements to 5000H and stores ABH (BL). SP is now 5000H. (Slide 139)

• POP DS Trace: If SP = 5000H holding ABH at 5000H and 10H at 5001H: reads word 10ABH into DS; SP increments to 5002H. (Slide 140)

• XCHG BX, AX Trace: Before: BX = 10ABH, AX = FF33H. After execution: BX = FF33H, AX = 10ABH. (Slide 141)

7.2 Arithmetic Instructions (Slide 142-145)

• ADD / ADC: `ADD AX, BX` (e.g., AX = 2000H, BX = 5000H -> AX = 7000H); `ADC` adds with Carry.

• SUB / SBB: `SUB Dest, Src`; `SBB` subtracts with Borrow (Carry flag).

• INC / DEC: `INC AX` (e.g., 7000H + 1 = 7001H); `DEC` decrements by 1. Note: Does NOT affect Carry Flag!

• MUL / IMUL: `MUL operand` (Unsigned) / `IMUL` (Signed). If operand is 8-bit, multiplies by AL and stores 16-bit product in AX (e.g., `MUL BH` with AL = 04H, BH = 02H -> AX = 0008H). If operand is 16-bit, multiplies by AX and stores 32-bit product in DX:AX. (Slide 145)

• DIV / IDIV: `DIV operand` / `IDIV`. 8-bit division: AL = Quotient, AH = Remainder. 16-bit division: AX = Quotient, DX = Remainder.

7.3 Logical & Bit-Shift Instructions (Slide 146-160)

• Bitwise AND: `AND AH, CH` (AH = F0H = 11110000, CH = 9EH = 10011110 -> Result = 90H = 10010000). (Slide 147)

• Bitwise OR: `OR AX, 0008H` (AX = 2000H -> AX = 2008H). (Slide 151)

• TEST: `TEST Dest, Src` performs bitwise AND to modify flags while operands remain unchanged.

• SHR (Shift Right): Shifts bits in register right by count; inserts zeroes from the left; rightmost bit shifts into Carry Flag (CF). (Slide 152-153).

• SHL / SAL (Shift Left): Shifts bits left; inserts zeroes at right; MSB enters Carry Flag. (Example: `MOV BL, 05H; SHL BL, 1` -> BL = 0AH). (Slide 154).

• ROL / ROR (Rotate without Carry): Rotates bits circularly left. (Example: `ROL AH, 01` shifts bit D7 into D0 and into Carry Flag). (Slide 155-158).

• RCL / RCR (Rotate through Carry): Rotates bits circularly left/right through the Carry Flag (9-bit circular loop). (Slide 159-160).

7.4 Control Transfer & Branching Instructions (Slide 161-164, 172-173)

• Unconditional Jump (JMP): Switches program execution to a target memory address unconditionally (`JMP 2050H`).

• Conditional Jumps: Transfers control only if flag condition is met: `JC` (Jump if Carry CF=1), `JNC` (Jump if No Carry CF=0), `JZ`/`JE` (Jump if Zero ZF=1), `JNZ`/`JNE`, `JS`/`JNS`, `JO`/`JNO`.

• Subroutines (CALL / RET): `CALL` pushes return address onto stack and branches to subroutine; `RET` pops return address back to resume main program.

• Interrupts (INT / IRET): `INT n` generates software interrupt; `IRET` returns from ISR (pops IP, CS, Flags).

7.5 String & I/O Instructions (Slide 165-171)

• String Definition (Slide 166): A series of data bytes or words stored in consecutive memory locations (DS holds source string, ES holds destination string).

• String Operations: `MOVSB`/`MOVSW` (move byte/word), `CMPSB`/`CMPSW` (compare), `SCASB`/`SCASW` (scan), `LODSB`/`LODSW` (load into AL/AX), `STOSB`/`STOSW` (store from AL/AX).

• Input/Output Instructions (IN / OUT): Ports act as hardware buffers (Slide 168). `IN AL, port` reads 8-bit data from port into AL; `OUT port, AL` writes data from AL to port. If port address > FFH, DX register is the ONLY register allowed to hold the 16-bit port address (`IN AX, DX`, `OUT DX, AX`). (Slide 169-171).

7.6 Flag Control & Machine Control Instructions (Slide 167)

• Flag Controls: `CLC` (Clear Carry), `STC` (Set Carry), `CMC` (Complement Carry), `CLD` (Clear Direction Flag), `STD` (Set Direction Flag), `CLI` (Clear Interrupt Flag), `STI` (Set Interrupt Flag).

• Machine State: `HLT` (Halts processor execution), `NOP` (No Operation), `WAIT` (Awaits external event/TEST pin), `ESC` (Escape to external coprocessor).

8. Addressing Modes of 8086

An Addressing Mode defines the way in which the operand of an instruction is specified and how the Effective Address (EA) in memory is calculated (Slide 174-187).

8.1 Detailed Breakdown of All 8 Primary Modes from Slides

1. Immediate Addressing Mode (Slide 176-178)

The operand data (8-bit or 16-bit) is specified directly within the instruction. Destination operand can NEVER be immediate data.

MOV CL, 15H ; 15H is transferred directly into register CL MOV BX, 1000H ; 1000H is transferred directly into register BX MOV AX, 10ABH ; 10ABH is loaded into AX (AH = 10H, AL = ABH) MOV AX, 2000H ; Immediate 2000H loaded into AX

2. Direct Addressing Mode (Slide 176, 179-180)

A 16-bit memory address (displacement/offset) is directly specified inside square brackets in the instruction. Default segment is DS.

MOV CL, [4321H] ; Moves byte from memory offset 4321H to CL MOV AX, [5000H] ; Moves 16-bit word at memory offsets 5000H (AL) and 5001H (AH) into AX ; If DS = 3000H, Physical Address = 30000H + 5000H = 35000H

3. Register Addressing Mode (Slide 176, 181-182)

Both operands are CPU registers. Data is stored in one register and copied to another (8-bit or 16-bit). All registers except IP can be used.

MOV AL, CL ; Copies 8-bit content of CL into AL MOV AX, BX ; Copies 16-bit content of BX into AX MOV AL, BL ; Copies BL into AL MOV AX, BX ; If BX = 10ABH, AX becomes 10ABH (AH = 10H, AL = ABH)

4. Register Indirect Addressing Mode (Slide 183)

The address of the memory location containing data is determined indirectly using an offset register (BX, BP, SI, or DI).

MOV AX, [BX] ; Offset is in BX. If BX = 5000H and memory at 5000H holds 33H, 22H, ; AX becomes 3322H (AL = 22H, AH = 33H). Default segment: DS.

5. Indexed Addressing Mode (Slide 184)

The offset of the operand is stored in one of the index registers (SI or DI). DS is the default segment.

MOV AX, [SI] ; Offset is stored in SI. If SI = 5000H, reads word at DS:5000H into AX.

6. Register Relative Addressing Mode (Slide 185)

The effective address is formed by adding an 8-bit or 16-bit displacement to the contents of any one of the registers BX, BP, SI, or DI. (Default: DS for BX/SI/DI, SS for BP).

MOV AX, 50H[BX] ; EA = [BX] + 50H ; If BX = 5000H, EA = 5000H + 50H = 5050H. ; Reads 16-bit data from memory offset 5050H into AX (AL = 44H, AH = 33H).

7. Base Plus Index Addressing Mode (Slide 186)

The effective address is formed by adding the content of a Base register (BX or BP) to the content of an Index register (SI or DI). Default segment is DS (or SS if BP is used).

MOV AX, [BX][SI] ; EA = [BX] + [SI] ; If BX = 1000H and SI = 2000H -> Final Offset EA = 1000H + 2000H = 3000H. ; Reads word at DS:3000H into AX (AL = 12H, AH = 34H).

8. Base Relative Plus Index Addressing Mode (Slide 187)

The effective address is formed by adding an 8-bit or 16-bit displacement with the sum of a Base register (BX or BP) and an Index register (SI or DI).

MOV AX, 50H[BX][SI] ; EA = [BX] + [SI] + 50H ; If BX = 1000H, SI = 2000H, Disp = 50H -> EA = 1000H + 2000H + 50H = 3050H. ; Reads word at DS:3050H into AX (AL = 12H, AH = 34H).

9. Implied / Implicit Addressing Mode (Slide 176)

The operand is implied directly within the instruction definition; no explicit operand is specified.

STC ; Sets Carry Flag (CF = 1) CLD ; Clears Direction Flag (DF = 0) CLC ; Clears Carry Flag (CF = 0)

8.2 Summary Table of 8086 Addressing Modes

| Addressing Mode | Effective Address (EA) Formula | Default Segment | Slide Example Syntax |
| --- | --- | --- | --- |
| 1. Immediate | Operand embedded in instruction | CS | MOV AX, 10ABH / MOV CL, 15H |
| 2. Direct | EA = [Displacement] | DS | MOV AX, [5000H] / MOV CL, [4321H] |
| 3. Register | None (Internal register) | None | MOV AL, CL / MOV AX, BX |
| 4. Register Indirect | EA = [BX / BP / SI / DI] | DS (or SS for BP) | MOV AX, [BX] |
| 5. Indexed | EA = [SI / DI] | DS | MOV AX, [SI] / MOV AX, [DI] |
| 6. Register Relative | EA = [Base / Index] + Displacement | DS (or SS for BP) | MOV AX, 50H[BX] |
| 7. Base Plus Index | EA = [Base] + [Index] | DS (or SS for BP) | MOV AX, [BX][SI] |
| 8. Base Relative Plus Index | EA = [Base] + [Index] + Displacement | DS (or SS for BP) | MOV AX, 50H[BX][SI] |
| 9. Implied / Implicit | Implicit in Opcode | Internal | STC / CLD / CLC |

Summary & Exam Review Checklist

This document synthesizes all lecture topics from the Sem III Computer Architecture and Organization course (ITC304):

✓ Module 1: Understand the 13 differences between Architecture and Organization, and the 3 CPU organization types (Single Accumulator, General Register, Stack).

✓ Module 2: Master the 5 functional units, CPU sub-parts, RAM vs ROM, and the 3 system buses (Data, Address, Control).

✓ Module 3: Memorize the 5 computer generations, key hardware components, and transistor density scaling.

✓ Module 4: Explain the Von Neumann stored-program model, Von Neumann bottleneck, Harvard architecture, and the complete 16-point CISC vs RISC comparison table.

✓ Module 5: Solve numerical problems using the Iron Law (T_CPU = N x CPI x T_clk), multi-class CPI equations, and explain pipelining / superscalar paradigms.

✓ Module 6: Draw and explain the 8086 internal block diagram (BIU vs EU), segment registers with physical address formula (PA = Segment x 10H + Offset), 16-bit flag register layout, and all 40 pins in Minimum vs Maximum mode.

✓ Module 7: Trace 8086 instructions across Data Transfer, Arithmetic, Logical/Shift, Branching, String, and I/O groups.

✓ Module 8: Formulate the Effective Address (EA) for all 8 primary addressing modes with assembly examples.